作业6 QSerialPort串口开发及应用（串口发送）

布置时间：5月3日

收集时间：5月10日24：00

提交方式：姓名_学号_作业6，如：平毅_2009060707_作业6.doc

内容：在Deepin操作系统（可在Windows版本上先进行开发）Qt Creator开发环境中设计一个QMainWindow类型的窗体。设置窗口大小为400X200分辨率，主窗口（标注应用名称为：“串口收发应用”，如图1所示），放置信息发送框（LineEdit），串口信息发送按钮（PushButton）以及串口信息接收框（TextEdit），按照参考样例布局。编写程序实现点击发送按钮，应用可发送发送框内所含字符串信息，该信息可通过虚拟串口工具（Virtual Serial Port）及串口助手查看(虚拟串口波特率需统一，设置为9600bps)，可参考图2应用示例。

图1 界面布局图（完整版）

图2 串口发送应用演示

解答：

1.新建Qt工程文件

2.mainwindow.ui文件中的QMainWindow对象进行分辨率（geometry参数）的设置

3.在mainwindow界面中添加TextEdit、LineEdit和PushButton控件，

4.插入头文件声明（在 mainwindow.h文件中插入private slots：）

private slots:

void on_pushButton_clicked();

void Read_Serial();

private:

Ui::MainWindow *ui;

QSerialPort *COMPORT;

QString Data_From_SerialPort;

bool IS_Data_Received = false;

5.添加源文件函数。

#include "mainwindow.h"

#include "ui_mainwindow.h"

MainWindow::MainWindow(QWidget *parent) :

QMainWindow(parent),

ui(new Ui::MainWindow)

{

ui->setupUi(this);

COMPORT = new QSerialPort();

COMPORT->setPortName("COM1");

COMPORT->setBaudRate(QSerialPort::Baud9600);

COMPORT->setParity(QSerialPort::Parity::NoParity);

COMPORT->setDataBits(QSerialPort::DataBits::Data8);

COMPORT->setStopBits(QSerialPort::StopBits::OneStop);

COMPORT->setFlowControl(QSerialPort::FlowControl::NoFlowControl);

COMPORT->open(QIODevice::ReadWrite);

if(COMPORT->isOpen())

{

qDebug()<<"Serial Port is Connected.";

qDebug()<<COMPORT->error();

}

else

{

qDebug()<<"Serial Port is Not Connected.";

qDebug()<<COMPORT->error();

}

connect(COMPORT,SIGNAL(readyRead()),this,SLOT(Read_Serial()));

}

MainWindow::~MainWindow()

{

delete ui;

}

void MainWindow::on_pushButton_clicked()

{

if(COMPORT->isOpen())

{

COMPORT->write(ui->lineEdit->text().toLatin1()+char(10));

COMPORT->flush();

qDebug()<<"Write Serial";

}

}

void MainWindow::Read_Serial()

{

if(COMPORT->isOpen())

{

while(COMPORT->bytesAvailable())

{

Data_From_SerialPort += COMPORT->readAll();

}

if(Data_From_SerialPort.at(Data_From_SerialPort.length()-1)==char(10))

{

IS_Data_Received = true;

}

if(IS_Data_Received = true)

{

qDebug()<<"Data From Serial Port: "<<Data_From_SerialPort;

ui->textEdit->append(Data_From_SerialPort);

Data_From_SerialPort="";

IS_Data_Received = false;

}

}

}

6.项目测试